Please find MQTT integration requirements for receiving telemetry data from and sending actions to our IoT telematics devices below.

  1. Connection Details

  The device supports two connection modes:

  Port 1883 (Plaintext)
  - No authentication required
  - Suitable for testing/development environments

  Port 8883 (TLS/SSL)
  - Mutual TLS authentication using device certificates
  - Requires a provisioning endpoint (see Section 2)

  ---
  2. Provisioning Server Requirements (for Port 8883)

    For TLS connections on port 8883, you must provide an HTTP provisioning endpoint that the device will call to obtain its configuration.

  Endpoint: GET /provision/{IMEI}/config.json

  Example Request:
  GET http://your-server.com:8080/provision/860123456789012/config.json

  The device will call this endpoint with its IMEI in the path. Your server must return the device configuration JSON with the mqtt block containing connection details and TLS certificates:

  {
    "mqtt": {
      "host": "mqtt.yourserver.com",
      "port": 8883,
      "device_id": "860123456789012",
      "tenant_id": "your_project_name",
      "keep_alive_sec": 60,
      "authentication": {
        "ca_certificate": "-----BEGIN CERTIFICATE-----\n...\n-----END CERTIFICATE-----",
        "device_certificate": "-----BEGIN CERTIFICATE-----\n...\n-----END CERTIFICATE-----",
        "device_private_key": "-----BEGIN RSA PRIVATE KEY-----\n...\n-----END RSA PRIVATE KEY-----"
      },
      "publish_intervals": {
        "can_parsed_ms": 1000,
        "device_shadow_ms": 10000,
        "gps_ms": 5000,
        "telematics_data_ms": 60000
      }
    }
  }

  Required MQTT Fields:
  - mqtt.host — MQTT broker hostname
  - mqtt.port — Broker port (1883 for plaintext, 8883 for TLS)
  - mqtt.device_id — The device IMEI
  - mqtt.tenant_id — Your project/tenant identifier (used in MQTT topic paths)

  Required for TLS (port 8883):
  - mqtt.authentication.ca_certificate — CA certificate for validating the broker (PEM format)
  - mqtt.authentication.device_certificate — Device's client certificate for mutual TLS (PEM format)
  - mqtt.authentication.device_private_key — Device's private key for mutual TLS (PEM format)

  Optional MQTT Fields:
  - mqtt.keep_alive_sec — MQTT keep-alive interval (default: 60)
  - mqtt.publish_intervals — Publish intervals for different data streams in milliseconds

  Notes:
  - Newlines in certificates must be encoded as \n within the JSON string
  - The endpoint must return HTTP 200 with Content-Type: application/json
  - The provisioning server should be accessible over HTTP (port 8080 by default)
  - The response can include other device configuration sections (events, gps, can, etc.) which will be merged with the device's base configuration
  - Maximum config size: 8KB

---
  3. MQTT Topics

  All topics use IMEI as the device identifier.

  Device → Cloud (Telemetry)
  /tenants/{project_id}/devices/{IMEI}/events/all_data/jsonarray
  Example: /tenants/revolt/devices/860123456789012/events/all_data/jsonarray

  The device publishes telemetry every 10 seconds (configurable).

  Cloud → Device (Actions)
  /tenants/{project_id}/devices/{IMEI}/actions
  Your backend should publish action commands to this topic.

  Device → Cloud (Action Responses)
  /tenants/{project_id}/devices/{IMEI}/action/status
  The device publishes action execution status to this topic.

  ---
  4. Telemetry Payload Structure (all_data stream)

  All data is published as a JSON array. The payload includes GPS, device shadow, telematics, and CAN parsed signals.

  [
    {
      "timestamp": 1704110400000,
      "sequence": 123,

      // GPS Data
      "latitude": 28.6139,
      "longitude": 77.2090,
      "altitude": 216.5,
      "speed": 45.2,
      "satellites_gps": 12,
      "satellites_beidou": 5,
      "satellites_used": 15,
      "fix_type": 3,
      "fix_status": 1,
      "pdop": 1.2,
      "hdop": 0.8,
      "vdop": 1.0,
      "azimuth": 45.6,
      "course": 180.5,

      // Device Shadow
      "status": "On",
      "battery_voltage": 3800,
      "external_voltage": 12000,
      "gps_fix_mode": 3,
      "modem_app_version": "2.5.1",
      "modem_base_version": "L511_base_v1.0",
      "controller_version": "2.0.0",
      "imei": "860123456789012",
      "iccid": "8910300000000000000F",
      "ble_mac": "AA:BB:CC:DD:EE:FF",
      "msisdn": "9876543210",

      // Telematics Data
      "gsm_status": 1,
      "gps_status": 1,
      "gps_ttff_s": 15,
      "imsi": "404123456789012",
      "apn": "internet",
      "csq": 25,
      "internal_available_disk_size_bytes": 1048576,
      "external_available_disk_size_bytes": 2097152,
      "modem_uptime_ms": 3600000,
      "gps_odometer_m": 12345.5,
      "can_odometer_m": 12340.0,
      "rpm_odometer_m": 12342.0,
      "external_battery_voltage_mv": 12000,
      "internal_battery_voltage_mv": 3800,
      "bluetooth_status": 1,
      "connected_device_name": "BLE_Scanner",
      "connected_device_mac": "AA:BB:CC:DD:EE:FF",
      "analog_input1_mv": 1500,
      "digital_output1_status": 1,
      "digital_output2_status": 0,
      "digital_input1_status": 1,
      "digital_input2_status": 0,
      "ignition_status": 1,
      "charging_status": 0,
      "controller_uptime_s": 1800,

      // CAN Parsed Signals (dynamic - depends on DBC config)
      "engine_rpm": 2500,
      "engine_coolant_temp": 85,
      "vehicle_speed": 45,
      "fuel_level_percent": 65,
      "throttle_position": 30,

      // Events (when triggered)
      "event_data_name": "ignition_on",
      "event_data_value": ""
    }
  ]

  Event Types: ignition_on, ignition_off, low_internal_battery, main_battery_disconnected, main_battery_connected, over_speed, harsh_acceleration, harsh_braking, harsh_turning, theft_detected, immobilizer_engaged, immobilizer_disengaged

  ---
  5. Sending Actions to Device

  Publish to: /tenants/{project_id}/devices/{IMEI}/actions

  Payload Format:
  {
    "id": "unique_action_id_123",
    "name": "action_name",
    "payload": { ... action-specific parameters ... }
  }

  The id field must be a unique identifier for tracking the action response.

  ---
  6. Action Response Format

  Subscribe to: /tenants/{project_id}/devices/{IMEI}/action/status

  Response Payload:
  [
    {
      "action_id": "unique_action_id_123",
      "timestamp": 1704110400000,
      "state": "Completed",
      "progress": 100,
      "errors": []
    }
  ]

  State Values:
  - Completed — Action completed successfully
  - Failed — Action failed (see errors array for details)
  - Downloading — OTA download in progress
  - Installing — OTA installation in progress

  ---
  7. Supported Actions

  7.1 set_digital_output

  Control digital outputs (relay/immobilizer).

  {
    "id": "action_001",
    "name": "set_digital_output",
    "payload": {
      "output_num": 1,
      "output_state": 1
    }
  }

  Parameters:
  - output_num (Integer) — Digital output number. Values: 1 or 2
  - output_state (Integer) — Output state. Values: 0 = OFF, 1 = ON

  ---
  7.2 set_total_odometer

  Set the total odometer value.

  {
    "id": "action_002",
    "name": "set_total_odometer",
    "payload": {
      "value": 50000
    }
  }

  Parameters:
  - value (Integer) — Odometer value in meters

  ---
  7.3 send_can_message

  Send a CAN message on the vehicle bus.

  {
    "id": "action_003",
    "name": "send_can_message",
    "payload": {
      "can_id": "0x7DF",
      "can_data": "0201050000000000",
      "is_extended_id": 0,
      "can_dlc": 8
    }
  }

  Parameters:
  - can_id (String) — CAN ID in hex format (e.g., "0x7DF")
  - can_data (String) — Data bytes as hex string (max 16 chars for 8 bytes)
  - is_extended_id (Integer) — 0 = standard 11-bit, 1 = extended 29-bit
  - can_dlc (Integer) — Data length code (0-8)

  ---
  7.4 update_config

  Update device configuration parameters.

  {
    "id": "action_004",
    "name": "update_config",
    "payload": {
      "gps_interval_ms": 2000,
      "telematics_interval_ms": 5000,
      "over_speed_threshold_kmph": 80
    }
  }

  Configuration is merged with existing settings. Keys not specified remain unchanged. The device will automatically reboot after a successful config update.

  ---
  7.5 ota_update

  Trigger firmware update over-the-air.

  {
    "id": "action_005",
    "name": "ota_update",
    "payload": {
      "modem_url": "https://example.com/firmware/modem_v2.6.0.bin",
      "controller_url": "https://example.com/firmware/controller_v2.1.0.bin"
    }
  }

  Parameters:
  - modem_url (String) — URL to modem firmware binary (optional)
  - controller_url (String) — URL to controller firmware binary (optional)

  At least one URL must be provided. Progress updates are sent via action status with states like Downloading, Installing.

  ---
  7.6 reboot_device

  Reboot the device.

  {
    "id": "action_006",
    "name": "reboot_device",
    "payload": {}
  }

  ---
  7.7 reset_device

  Soft reset the device.

  {
    "id": "action_007",
    "name": "reset_device",
    "payload": {}
  }

  ---
  8. Acknowledgment Requirements

  The device uses MQTT QoS 1 for all publish operations. Your broker must properly acknowledge messages.

  - QoS Level: 1 (At least once delivery)
  - ACK Timeout: 15 seconds (configurable)

  Important: Once the device receives a PUBACK from the broker, it assumes the server has successfully received and persisted the data. The device will then discard the message from its local buffer. Your backend must ensure messages are durably stored before the broker sends the ACK.

  ---
  9. Important Notes

  1. Action Queuing: Only one action is processed at a time. If an action is in progress, additional actions will be rejected with an error response.
  2. Action Retries: Actions such as OTA updates can fail intermittently due to network connectivity issues, server unavailability, or other transient errors. Your backend should implement retry logic based on the error message returned in the action status. Common retryable errors include:
    - Network timeout during download
    - HTTP errors (5xx) from firmware server
    - Connection reset during transfer
  3. Timestamps: All timestamps are Unix epochs in milliseconds.
  4. Sequence Numbers: Each telemetry message includes a sequence number for deduplication on your end.
  5. CAN Data: CAN parsed fields are dynamic and depend on the DBC configuration loaded on the device.
  6. Subscription Patterns: Your backend should subscribe to:
    - /tenants/{project_id}/devices/+/events/all_data/jsonarray (for telemetry from all devices)
    - /tenants/{project_id}/devices/+/action/status (for action responses from all devices)

7. MQTT Server setup, maintenance and uptime needs to be managed by you. Dashboard development also needs to be managed by you.